# Induction Coil — Run Procedure

This covers building, flashing, and safely running the induction coil control (PWM + deadtime + button-driven heating + safety cutoffs). It’s a companion to docs/README.md (the general self-test flow) and docs/protocol.md (the UART wire format) — this one is specifically about the coil/heating feature.

## 1. Build & Flash (CCS)

- Open Driverlib Empty CPU1 Example CCS Project in Code Composer Studio.

- Clean the project.

- Build. Check the build Output/Console for Finished building target: "...out" with no compile/link errors — trust that over the Problems tab, which can show unrelated false positives.

- Debug (flashes the board, opens a debug session — paused, not running yet).

- Click Resume (▶ / F8). This step is easy to miss — without it the program is loaded but not actually executing.

## 2. Boot sequence — what to expect

- The board runs a current-sensor calibration sweep on boot, before anything else is armed. This can take up to ~1 minute.

- Do not press any button during this time — nothing is listening yet.

- The board signals it’s actually ready two ways, at the same moment:

  - A long buzzer tick (~200ms) — audible, no screen needed.

  - The text READY sent over UART (visible in the GUI’s status bar as “Board ready”, or in the Debug Console — see below).

- Wait for one of these before touching any button. This is the #1 cause of “I pressed the button and nothing happened.”

## 3. Connecting the GUI

- cd gui-app && python3 app.py

- Select the board’s serial port, click Connect.

- Wait for the status bar to read “Board ready.”

## 4. Button behavior

| Button | Effect |
| --- | --- |
| ButtonLOW / ButtonMEDIUM / ButtonHIGH | Starts heating at that mode — only works if heating is currently OFF. Also closes the DC bus contactor (relay) as part of the same action. |
| ButtonON | Stops heating (from HEATING or an ERROR state) and opens the contactor. Does not start heating — pressing it while OFF does nothing. |

Two audible signals confirm what happened on any press:

- Short tick (10ms, buzzer) — fires on every genuine press, whether or not it actually changed anything. This confirms firmware saw the press.

- Click (relay) — fires only when the contactor actually opens/closes, i.e. only on a real start/stop transition.

Reading the combination: - Tick + click → button worked, heating state actually changed. - Tick, no click → firmware saw the press but refused it (see Diagnostics below for why — most likely calibration or already-in-that-state). - No tick at all → firmware never saw the press (board not ready yet, or a wiring/GPIO issue — not a logic problem).

## 5. Safety behavior

- Deadtime: 100ns gap enforced by the PWM hardware itself between the two switch outputs, on every switching edge, always active — not something that can be accidentally skipped or turned off in normal operation.

- PWM: fixed 45kHz, ~50% duty cycle. No frequency/resonance tracking (“angle finding”) — deliberately not implemented, per team direction (see §6 below).

- 30-minute auto-shutoff: heating force-stops on its own after 30 minutes even if nobody presses anything.

- Over-current trip: a hardware comparator (CMPSS) trip is polled every 1ms while heating. On trip, heating force-stops, the contactor opens, and the board enters an ERROR state that only clears via ButtonON.

## 6. Diagnostics — Debug Console

The GUI has a Debug Console box at the bottom (text field + Send button, with a log underneath). No separate terminal app needed.

- Type COIL_STATUS, press Enter → returns current state (OFF / HEATING / ERROR), heat mode, the calibrated current-sensor value, and whether that calibration is VALID or INVALID.

- Type BUTTON_STATUS, press Enter → returns how many genuine presses firmware has counted per button since boot. Press a button, query again, compare the count — confirms whether the press was actually detected.

## 7. What’s intentionally NOT included (and why)

- Angle-finding (dynamic frequency/phase tracking to chase resonance) — skipped per team direction. Heat mode (LOW/MEDIUM/HIGH) is tracked for reporting but doesn’t currently change the switching frequency, since that differentiation only ever came from the angle-finding logic being skipped.

- Full RMS/peak current estimation (the original reference design’s DMA + ADC sine-capture pipeline) — not ported; this project’s over-current protection is comparator-trip-based only (see §5), which is real protection but a different mechanism, not a drop-in equivalent.

- HMI status messages / a separate debug UART frame protocol — not applicable to this test fixture, which uses its own existing self-test UART protocol (docs/protocol.md) instead.

## Quick checklist

- Clean → Build → Debug → Resume

- Wait for the long tick / “Board ready” — not a fixed time estimate

- python3 app.py, Connect

- Press a heat button → listen for short tick + relay click

- If only a tick, no click → type COIL_STATUS in Debug Console, check why

- Press ButtonON to stop → listen for short tick + relay click (opens)